Universal High & Low Voltage Power Supply.

This power supply will provide high-voltage that could be used for Screen Voltage,

Or Plate Voltage along with bias voltage and low voltage that may be used for relays or other 12 DC accessories.

The High Voltage section uses eight 1n4007 diodes to make up the full wave bridge circuit.

The 3 snap in High Voltage electrolytic Capacitors no larger then 25 MM in diameter, there are three Solder pads on the bottom side of the board for equalizing/bleeder resistors for the High Voltage section, Ratings can be adjusted to suit your needs.

Note the BIAS and Low Voltage snap in electrolytic capacitors may not be larger then 20 MM

In diameter, If larger the diodes will be covered up!

The BIAS section I used a full wave Center Tap, the user could use just a Half wave rectifier circuit simply by installing only 1 Diode.

Beneath the BIAS electrolytic capacitor is also a solder pad for a bleeder resistor.

The Low Voltage section is a full wave rectifier circuit (4 diodes) along with an electrolytic Capacitor of your choice based on your voltage requirements, relay or other sources.

Mike Melnik N9PMO